EXHIBIT “C”

to all

SUBCONTRACT AGREEMENTS

Made by

KIER CONSTRUCTION CORPORATION

# DETAILED SCOPE OF WORK

PROJECT:	PROJECT NAME

SCOPE:	DIVISION 01 – GENERAL CONDITIONS – AS IT MAY APPLY

DIVISION 09.9123 – INTERIOR /EXTERIOR PAINTING

SUBCONTRACTOR:	COMPANY NAME

COST CODE: 00-0000

- PAINTING AND COATINGS:				$000

- WALL COVERINGS					$000

- FINAL TOUCH-UP OF TRADE DAMAGE		$000

- TOTAL SUBCONTRACTED AMOUNT:   		$000

CONTRACT DOCUMENTS:

Includes all labor, materials and equipment necessary for complete Painting scope for the PROJECT NAME. All work shall comply with all plans and specifications:

  - DATE BID SET

  - DATE SPECIFICATIONS

  - DATE BID SET RFI #1

  - GEOTECHNICAL REPORT DATED DATE

SCOPE OF WORK:

- Subcontractor to provide labor and materials to paint all drywall, trim, baseboard, unit entry doors (both sides), unit shelving, closet doors, hand/guardrails, and all common areas per plans and

- specifications.

- Provide and install Corridor painting and Common area painting; sheen, colors, and tones are to be per Interior Design drawings and specifications. In locations where 2 walls meet, this subcontractor is responsible for ensuring the break line between the two colors is pristine, clean, straight, and true. All sheens and colors are to be in accordance with the ID set of drawings.

- Provide and install painting throughout Units with unit Entry Door accent. All sheens, colors, and tones are to be in accordance with the ID set of drawings.

- Provide and install all common area wall coverings per Interior Design drawings and specifications.

- Subcontractor to accommodate and include 2 hours of touch up per unit on the job; tickets of work performed must be provided to Kier site superintendent daily of work completed.

- Provide and install 40 hrs. minimum of touch-up for corridors & amenity spaces. Tickets of work performed must be provided to the Kier site superintendent daily of work completed.

- Paint shall be primer and then 2 coats per plans and specifications.

- The subcontractor shall provide all finish necessary, including but not limited to sanding prep surfaces, primers, sealers, caulking, and puttying for trim, millwork, shelving, baseboards, hollow metal frames, and doors.

- Subcontractor shall sand walls after primer before painting.

- The subcontractor shall, in accordance with the contract documents, paint all fire extinguisher cabinets, HVAC grilles, ductwork, diffusers, access panels, plumbing, conduit, and electrical panels in gypsum board walls and ceilings to match adjacent wall finish.

- Includes painting all unit entry and common area doors. Includes painting of the exterior side of all exterior doors.

- Any exterior doors and trim, brick mold, garage louvers, grills that are painted before installation, etc., to be painted by this subcontractor.

- The subcontractor shall field paint exposed bare and covered pipes, ducts, hangers, steel, and iron work, and primed metal surfaces of equipment installed under mechanical and electrical work.

- The subcontractor shall provide all prep with exposed pipes, ducts, hangers, steel, iron metal, etc., including all etching and bonding primers as required.

- Subcontractor shall keep all fire rating labels free of paint.

- Subcontractor shall paint pipe bollards as identified and required by contract documents.

- Before the start of painting, the Subcontractor has the responsibility to verify the drywall finish and other surfaces or substrates scheduled to be painted prior to acceptance of the finish. The subcontractor will notify the contractor at least 48 hours before the scheduled start of any areas requiring remedy before proceeding. Painting surfaces implies acceptance of substrate and acknowledgment that work will be performed to the job-specific requirements outlined in the contract drawings and specifications. The final wall finish will be the painter’s responsibility.

- Includes caulking all interior dissimilar materials to drywall (i.e., millwork to drywall, windows to painted gypsum, etc.), including the top of door casing for blower door testing.

- Subcontractor shall provide caulking of all joints, seams, and corners where they occur at a painted surface. Including but not limited to the door stop-to-door jamb, door jamb to the casing, casing to drywall, closet shelves to apron support, apron support to drywall, etc.

- The subcontractor shall provide caulking of painted baseboards to floors; the subcontractor shall also caulk the door jambs and the door casings to the finished floor.

- Subcontractor shall caulk the drywall to the electrical boxes, outlets, lights, etc.

- Includes painting all patio doors, entry doors, hollow metal doors and frames, lite kits, side doors, and sidelights. The subcontractor shall provide a painting of all storage and mechanical room doors and walls. All sides of doors shall be painted, including jambs, head, bottom, brick molds, casings, and door edges.

- Includes all touchup and sheetrock repair following commencement of finishing trades.

- The subcontractor shall provide a punch-list person and/or crew during the final deficiency resolution and owner inspection period for touch-up and repair.  This person/crew shall be separate from the contract production work crew.

- Provide task lighting as needed during construction for your specific trade.

- The subcontractor shall take special care in caulking and masking the tub-shower surround to ensure a smooth and attractive finish, with a crisp line where the painted wall meets the shower edge.

- The subcontractor shall provide daily cleanup of all related debris and placement of debris in the dumpster at the site.

- Subcontractor shall always provide an English-speaking foreman on the job.

- Subcontractor shall also keep the same crew on the site to man the project from start to finish.

- The subcontractor shall ONLY wash out brushes, rollers, etc., in an area designated by the Project Superintendent. NO WASH OUT OF BRUSHES/ROLLERS INTO UNIT PLUMBING FIXTURES IS ACCEPTED. Any damage done to the unit or common area fixtures done by this Subcontractor shall be repaired or replaced at no additional cost to the Contractor or Owner.

- Subcontractor shall provide a paint summarized on the approved manufacturer’s list, as shown in the specifications.

- In accordance with Specification 09-90-00, the subcontractor shall paint 2” stripes on the stair nosings that do not have contrasting colors.

- In accordance with the specifications, the subcontractor shall provide repair, at the completion of work of other trades, touch-up and restore damaged surfaces or defaced painted surfaces.

- Subcontractor shall use CMU Filler on any CMU surfaces where CMU is required.

- Subcontractor shall provide a plan for removing weather stripping, and re-installing weather stripping after painting is completed. Any weather stripping not re-installed after painting that is then lost will need to be furnished by this subcontractor.

- Subcontractor shall provide epoxy paint where paint is the finish behind mop/ janitor sinks and any other wet areas, as required by code.

- The subcontractor shall clean all overspray/paint drips on finish surfaces

- Subcontractor shall provide protection of all floors, including Gypcrete/Concrete, from overspray for LVT installation. (All Flooring, Base, & Case are generally to be installed before painting)

- Subcontractor shall fully mask concrete floors that are to be left exposed.

- Re-hanging any doors removed during the painting process.

BUILDING SCHEDULE DURATIONS:

The subcontractor acknowledges that sufficient forces will be provided for the entirety of the work as this scope of work is required on the project.

Detailed Schedule Breakdown:

- PREP/PAINT IN UNITS 		7 UNITS PER DAY

- PREP/PAINT HALLWAYS 		3 DAYS PER FLOOR

- TOTAL DURATION: